DIC HW5

312510190 張家瑋

PART1: Implement a 3x3 convolution kernel without clock gating.

實現convolution without clock gating, clock period 滿足設定之1000ps

面積小於5100um2。

PART2: Use the pattern provided by TA to generate the waveform under gate-level simulation, and measure the power consumption of 3x3 convolution kernel using PrimePower

(1). Without using clock gating

waveform:

根據power report，dynamic power佔了整體99.8%左右，其中Internal power又佔了dynamic power的90%以上，這代表整個電路主要的功耗來自於電晶體切換時NMOS、PMOS同時導通所造成的Short circuit power。

Power consumption的來源，又以register為大宗，combinational則為其次。

PART3: Power reduction using a data-driven clock gating technique

設置CG元件，由於0乘上任何數字都是0，因此當In_IFM從一個不為零的數字變成0的時候，我們可以把In_IFM_reg的clk給關掉，相加的時候直接輸入0，這樣可以有效減少register切換的次數進而節省power。

(2). Using clock gating

使用CG後的power降到120uW。

PART4: Compare and analyze the area and critical path

|  | area | power |
| --- | --- | --- |
| Without CG | 4809um2 | 207uW |
| With CG | 5525um2 | 122uW |

使用CG後，雖然area增加約14.8%，但power卻節省41.1%，可發現CG確實大大增加功率效益。

根據公式Pinternal power=tsc·Vdd·Ipeak·fclock，internal power能降這麼多的原因主要是tsc(NMOS PMOS同時導通的時間)下降很多，也就是暗示register切換(0→1 or 1→0)的次數明顯減少。

Critical Path Without CG:

根據timing report，critical path來自於IFM_3_reg_1到ans_reg_18的路徑，

Critical Path With CG:

根據timing report，critical path的起點在Weight_3_reg_1，終點在ans_reg_17，如圖所示

還沒gate的時候，critical path會從IFM出發，但當使用CG後，critical path起點是從weight_reg開始。